ECE 383 Final Project Plan – Brandon Sweitzer

Tetris

- Hardware Organization

The game will be implemented as a modular, synchronous VHDL design, organized into separate components that interact through clearly defined signals and a top-level controller.

The Top-Level Structure will consist of:

- Input Subsystem for the NES controller interface

- Game logic subsystem including the FSM, game board, and piece logic

- VGA video output

- Timing with clock dividers to establish the game speed

- Major Hardware Modules

- NES Controller Interface

Reads and decodes player input from NES controller.

Interface will need to:

- Generate latch/clock signals

- Decode button states

- Take serial button data

- Output debounced button signal

- Game state controller

Control flow of the game using FSM with the following states:

- Init

- Idle

- Spawn

- Fall

- Move

- Lock

- Clear

- Game_over

State will determine when pieces move, lock in place, or disappear when a row is cleared as well as progression for score and levelling.

- Grid Storage

Store current state of the Tetris grid handled by a 2D array/grid stored in BRAM or registers where each cell will be marked as occupied/unoccupied.

Grid storage will be able to read current board state for video output, update the board when pieces lock at the bottom, and shift blocks down when rows are cleared.

- Piece Logic

Manages actively falling pieces. Piece logic will have to:

- Generate new pieces

- Track piece position

- Track rotation

- Output coordinates for collision logic and video rendering

- Collision Detection

Ensure piece moves are valid by checking wall boundaries, floor boundaries, and blocking overlap with preexisting blocks.

This module will have to:

- Block illegal moves

- Output when a piece is ready to lock

- Line Clear Logic

Detect and delete full rows. Clear logic will have to:

- Scan board for full rows

- Clear rows and shift higher rows down

- Update game score

- VGA Output

Display game visuals, use Lab 1 code to make:

- Grid

- Active piece

- Locked pieces

- Game UI

- Timing

Controls game speed using clock dividers to:

- Set piece falling rate

- Generate input sampling for controller

Block Design:

FSM: